| 1 | Install the usb driver https://ftdichip.com/drivers/d3xx-drivers/ 1.3.0.4 WHQL Certified |
| --- | --- |
| 2 | Install the pothosSDR dll drivers https://downloads.myriadrf.org/builds/PothosSDR/ PothosSDR-2021.07.25-vc16-x64.exe Ensure that the paths are added. E.g. installed location is at C:\Program Files\PothosSDR Search the “Edit the system environment variables” Click environment variable Select Path under user variables Ensure that C:\Program Files\PothosSDR\bin is in the path Ensure that system variables are in Enter the environment variable SOAPY_SDR_PLUGIN_PATH C:\Program Files\PothosSDR\lib\SoapySDR\modules0.8 |
|  | Plug in the LimeSDR to run the quick diagnostic test that the sdr can be connected and recognized PS C:\Users\65844\Desktop> LimeQuickTest [ TESTING STARTED ] ->Start time: Sun Mar 20 13:02:35 2022 ->LimeSuite version: 20.10.0-PothosSDR-2021.07.25-vc16-x64 ->Device: LimeSDR Mini, media=USB 3, module=FT601, serial=1D58972726D5C9, index=0, HW=3, GW=1.30 Serial Number: 1D58972726D5C9 Chip temperature: 39 C [ Clock Network Test ] ->REF clock test Test results: 26816; 40015; 53211 - PASSED ->VCTCXO test Results : 6711006 (min); 6711160 (max) - PASSED ->Clock Network Test PASSED [ FPGA EEPROM Test ] ->Read EEPROM ->Read data: 13 0A 17 13 0A 17 02 ->FPGA EEPROM Test PASSED [ LMS7002M Test ] ->Perform Registers Test ->External Reset line test Reg 0x20: Write value 0xFFFD, Read value 0xFFFD Reg 0x20: value after reset 0x0FFFF ->LMS7002M Test PASSED [ RF Loopback Test ] ->Configure LMS ->Run Tests (TX_2 -> LNA_W): CH0 (SXR=1000.0MHz, SXT=1005.0MHz): Result:(-18.3 dBFS, 5.00 MHz) - PASSED ->Run Tests (TX_1 -> LNA_H): CH0 (SXR=2100.0MHz, SXT=2105.0MHz): Result:(-20.1 dBFS, 5.00 MHz) - FAILED ->RF Loopback Test FAILED => Board tests FAILED <= Elapsed time: 6.13 seconds |
|  | Restart PC? |
|  | Install miniconda with python 3.7 https://docs.conda.io/en/latest/miniconda.html Pick the version 3.7 only, not the latest. |
|  | If miniconda is installed here: C:\Users\65844\miniconda3 Copy the pycitrus package (pycitrus.cp37-win_amd64.pyd) to the location: C:\Users\65844\miniconda3\Lib\site-packages |

Developer notes: